27 July 2022

Jeffery Alexander McTainsh and Gwendoline Mary McTainsh

C/- First Homes (Bay of Plenty) Limited

1 Aeropark Way

Mount Maunganui  3116

| Reference Number: | EBC-2022-1634/0 |
| --- | --- |
| Property Address: | 9 Dune Rise, Whirinaki 0473 |
| Property ID # | 3362412 |
| Description: | Foundations for Prefabricated Transportable Dwelling and Install On-Site Wastewater System |

Dear Sir / Madam,

Further Information Request - Building

Work on your application has been suspended because further information is required to demonstrate compliance with the New Zealand Building Code. Processing of your application will resume on receipt of all of the information listed below:

- LBP number provided in Certificate of Work is not associated with Logan Roberts or Lightbulb Architecture.  Please provide new Certificate of Work with correct LBP number.

- Site suitability report stating that considering soil properties, all proposed lots are subject to further geotechnical investigation and specific foundation design at the time of building consent.  Please provide site specific geotechnical investigation along with a foundation design from a suitably qualified engineer, and accompanying PS1.

- Design indicates timber piles, however Sheet 10 notes raft slab design.  Please remove irrelevant information.

- Proposed dwelling is to be located in corrosion zone D.  Please specify fixings that are suitable for the corrosion zone.

- Please provide information on subfloor ventilation, crawl space and subfloor access to comply with building code.

- Deck is Exempt under Schedule 1, however legal access into the building still required.  Please provide detail of legal access including tread, risers and handrail where necessary.

- Please provide information on the type of secondary treatment system proposed for this project.

- Wastewater report provided specifies dripper lines at both 0.5m crs, and 1m crs.  Author of the wastewater report to confirm spacing of dripper lines and revise pages 3, 4, 14 and drawing DE1 for consistency.

- Please provide location of smoke detectors to comply with F7 and update plans.

To reduce further processing costs and delays, please email ALL the listed information in one response to bsg@fndc.govt.nz.

If you are a registered customer, select the link below to provide this information:

https://online.fndc.govt.nz/ePathway/Production/Web/GeneralEnquiry/ExternalRequestBroker.aspx?Type=L-BDAP&Module=EGELAP&Class=BUILD&ResponseType=FINFO&ApplicationId=937185&DocumentId=4463795&ForceLogin=true

If there are good reasons why you cannot supply this information, please contact us urgently. We may be able to assist or arrange an extension of time.

We will hold your application for 20 working days from the date of this letter. If we do not hear from you or receive the outstanding information in that time we may refuse the application.

Should you have further questions please contact the building team on 0800 920029 or 09 401 5200 or email us at bsg@fndc.govt.nz.

Yours faithfully,

pp

Victoria Cree

Building Control Officer

District Services

Emailed to: julia@firsthomes.nz